- About Laser
How is laser engraving and laser cutting realized?
：The laser light to burn the material and the intensity of the burn determines whether it leaves a mark or burns through the material.
Engraving and cutting are realized by the speed and power settings for the same material.
Reference parameter：
Annex 1

- caveat：
Storage environment：
Please ensure that the machine is stored in dry, well-ventilated temperatures,（there is a risk of damage to the motherboard if it is left on a wet floor for a long period of time.）

Safe to use：
Please ensure that the environment is ventilated, wear the glasses, and wear a gas mask as far as possible when engraving acrylic and other polymer materials.

Regular cleaning：
Module cumulative work every 8 hours, use alcohol cotton to clean the bottom of the module window lens. Ensure the normal power and life of the module.
Engraving risk：
Annex 2
Pixi laser engraving reflective materials (such as part of the stainless steel, coated mug) when there is a risk of damage, Please refer to the attachment: Metal Engraving Guide for engraving.

- pixi engraving anomaly adjustment method
Annex 3

- WiFi connection and APP use
Annex 4

- Software Installation and Tutorial
Annex 5

- Learning and Using AlgoOS Functions
Annex 6

Will the module power be reduced?？

- Long continuous working time of the module causes the internal temperature to rise, which affects the output power. The power is restored after resting to normal temperature.

- Long time did not clean the module bottom lens or window lens consumption damage, resulting in part of the power absorbed by the lens. At this time the lens needs to be cleaned or replaced.

- For more FAQ, please check the AlgoLaser official website
https://algolaser.com/pages/faq